# Introducing regular climatic investigations of the solar ultraviolet radiation in Sofia

G. Petrunov, N. Miloshev, L. Draganov, G. Miloshev

Geophysical Institute, Bulgarian Academy of Sciences, Acad.G.Bonchev Str. bl.3, Sofia1113, Bulgaria, e-mail: miloshev@geophys.bas.bg

Abstract.  The beginning of regular instrumental investigations and measurements  of the solar ultraviolet radiation in Sofia are presented in this paper. The constructed instrumentation (apparatuses) and the used methods are described in details. The first results for the year 1997 are graphically presented. The future scientific and practical applications of the obtained results are pointed out in the conclusion.

Key words: solar ultraviolet radiation, UV-A radiation, UV-B radiation, UV measurements, UV forecasting.

Introduction

Ultraviolet rays were some of the factors in life origin on the earth and also a necessary condition for existing, development and improvement of living nature. They are one of the components of the unity between the physical conditions on the planet and the biological world. In the history of mankind evolution man has experienced the beneficial action of ultraviolet rays. At the same time the human system has built its own protection against their excessive action.

In the earth’s atmosphere nowadays phenomena are observed which considerably influence the transport of solar ultraviolet radiation (UVR) through the atmospheric layer to the earth’s surface. As a result the UVR intensity and spectral composition are changed resulting to the disturbance of the balance Man-UV rays which has been built in the course of million years. The changes occurring in the ozone layer, connected with the decrease of ozone concentration in the atmosphere high layers, lead to an increase of the intensity and lowering the short wave boundary of the solar UVR Bojkov (1992), Bojkov (1995). Initially affected regions have been found out in the southern hemisphere and later in the northern hemisphere of the Earth. The prediction for a further unfavorable development of these changes, over the geographic latitudes of Bulgaria as well, determines the introduction of the solar UVR monitoring on a world scale. This is a pressing task for physics of the atmosphere, the results of which would also help the improvement of the physical basis for a rational use and dosage of the UVR natural resources that are extremely unevenly distributed both in territory and time.

The UVR has a powerful biological influence. This is due to the fact that from the whole range of optical emission UVR has the highest quanta energy, which is delivered to the absorbing molecule. The UV rays possess prophylactic and therapeutic action, their effect on the human system is generally favorable, they increase the vital energy, contributing to the system steadiness to infection diseases. At an optimal dosage the UV rays render a definite beneficial influence on the living systems. It is proved that the deficiency or complete lack of natural UVR result in pathological changes of the human body called “light hunger” or “UV deficiency”. On the other hand the systematic excessive radiation has too unfavorable consequences. They are most strongly expressed on skin and eyes. The high degree of damage can cause skin cancer. Conclusions of experts based on scientific investigations indicate that the ozone layer thinning with 1% results in an increase of melanoma cases by 5-6% (Urbach et al., 1986), (Peak et al.,1987), (Robertson 1982). The consequences of the excessive radiation on the plant and animal world are not less heavy and dangerous (Caldwell et al., 1986), (Mitchell 1990). The negative impact of the increased doses of solar UVR on man and all biological systems is also indirect – for example these rays cause the increase of smog toxicity in the regions with great industrial concentration.

From the above stated it becomes clear how pressing the problem of the regular measurement and investigation of the solar UVR is. It is of both purely scientific and also applied value. For this country the investigations on the intensity and spectral composition of the solar UVR have also practical importance evolving from the fact that Bulgaria is a country where agriculture and tourism are national priorities. Both branches are connected with the continuous stay of people under the influence of the natural UVR. This necessitates the elaboration of methods for a dozed and safe use of this natural factor.

Nowadays, following the recommendations of the World Meteorological and Health Organizations, in a number of countries continuously operating stations and networks of stations for investigating the solar UVR are created. Profound studies on the intensity and the spectral composition are carried out as well as on the factors considerably influencing the radiation increase or decrease. Beside diagnostic recently these stations have also prognostic purposes, namely to predict the intensity of the solar UVR and to warn about any dangerous excess above the admissible values (Report of WMO, 1994), (Report of WMO-WHO, 1997), (Lesgegynski et al., 1993).

Making Base Station For UVR Registration

Guided by the importance of the problem the authors established a base actinometric  station for investigating the sun UVR which in perspective will serve as a methodical center for the development of a national network of some more at least 2 or 3 similar stations at a different above sea level height. Such a network of stations would give an opportunity in a long-term plan to characterize the UV climate of Bulgaria. The accumulation of a sufficient massif of data is also a necessary condition for taking into account the specific influence of the large group of factors considerably influencing the UVR regime.

The station is situated on the observation site on the roof of the Geophysical Institute at the Bulgarian Academy of Sciences. It meets the necessary requirements, namely: height above the earth surface about 20 m, open horizon, lack of shading or reflecting objects or buildings.

As a meteorological factor the solar UVR, its intensity and spectral composition depend mainly on the following factors:

- height of the Sun during the day

- the season

- the above sea level height

- transparency of the atmosphere (cloudiness, fogs, aerosols, gases)

- Albedo of the Earth’s  Surface.

The main factor influencing both the intensity and the spectral composition of UVR is the Sun’s height above the horizon. It accumulates in itself the influence of the total ozone content and the atmosphere transparency.

The influence of the above sea level height on the UVR intensity can be investigated when at least two more stations are available – at a sea level and at about 2000-2500 m above sea level height. This will be a future stage of the development of a national network, which is our goal. Besides everything else a high mountain station gives an opportunity to investigate the albedo role under winter conditions. It is known that fresh snow reflects up to 90% of the intensity of the falling  UVR rays which is a reason for extremely severe skin and eye damages.

Instrumentation (Apparatuses)

The measurement of the sun UVR is a complex problem from a physical and technical viewpoint. The UVR comes to the earth surface as an insignificant part – about 3% of the powerful solar emission in the visible and infrared region of the spectrum. And in the spectral region l<315 nm not more than 0,05%. First, the filtration and measurement of such a negligible quantity of energy is accompanied by high technical requirements. Second, the energy and the spectral composition of the solar UVR continuously vary in wide boundaries as a function of the sun height and the meteorological conditions.

Because of the lack of expensive industrial measuring apparatuses, the authors constructed a “Photoelectric UV meter for an integral measurement of the solar UVR” in the spectral regions l<380 nm and l<320 nm.

The UV meter consists of receiver – optical block and measuring unit – electronic block. The optical block is formed of two photo sensors filled in on the basis of antimon-cesium  photocells and the respective filter systems for the two spectral ranges. They are equipped with an appliance for correcting the UV irradiation by Lambert cosine law.

The photocells used have a weak temperature dependence. They preserve their qualities in the course of years.

The receivers are air-proof and mounted stationary on the aktinometric site. By means of a water-proof cable they are connected with the electronic block situated in the building.

## Methods

The aim of the investigation is to record the diurnal and yearly course of the UVR intensity and the setting in of changes as a result of the processes in the ozonosphere. All this requires long term, regime, instrumental observations. By means of the described formulation the intensity of the total solar UVR on a horizontal surface is measured. It is formed by the intensity of the direct solar UVR and the secondary, dissipated UVR which comes from the atmosphere. The measurements are made every 30 minutes. Such an interval of time gives the necessary quantity of data for a sufficiently precise description of the UVR diurnal course. The instrumentation gives a possibility for shortening the interval of measurement to 30 sec. This is applied when there is a fast changing cloudiness or in the early morning or late afternoon hours when the gradient in the intensity change is too big. The data from the measurements of the solar UVR are recorded in protocols and then introduced in computers for automated storage and processing. They can be in a table or graphic form. So that the data from different days and seasons measured by hours in civic time are comparable, it is necessary they to be reduced to the corresponding height of the sun in grades for the moment of measurement. To this end a computer program was worked out which presents the opportunity to determine the sun’s height in degrees for each minute of the civic time. The UVR intensity measurement is indirect, in relative units, by means of the intensity of the photo current coming from the photocells. The emissions of the photocells are of the order of Ax10-9 for the region l<320 nm and of Ax10-7 for the region l<400 nm.

Results

In the station at the Geophysical Institute from the month of May, 1997 daily measurements of the solar UVR  in the  defined  spectral  ranges  are  carried out. The accumulating massif of data gives an opportunity to obtain information about the diurnal and yearly courses of the solar UVR intensity. At the same time additional statistical elaboration will allow the establishment of a series of correlation, for example with the quantity of ozone, the atmosphere transparency, etc. The UVR intensity is measured in a photo current  within the ranges Ax10-7 for l <380 nm and Ax10-9 for l<320 nm. For illustration the following is presented: the intensity diurnal course for characteristic summer and winter months (June and December) as a function of the sun’s height on a cloudless day, after a front and precipitation and for a day with changeable cloudiness for the two spectral ranges respectively. On each of the figures it is clearly seen what a serious in quantitative respect barrier is cloudiness. At the same time it becomes apparent what an insignificant part of the total UVR, is its biologically active part with wave length smaller than 320 nm. The comparison also shows the big (almost four-fold) change of UVR intensity depending on the Sun’s height.

Figure 1.  Daily course of the UV-A (<380nm) and UV-B (<320nm) for a sunny (dashed line) and cloudy (solid line) day of June and December 1997.

## Conclusions

- A base aktinometric station for investigating the solar UVR at the Geophysical Institute of BAS is established and equipped.

- Instrumentation devices for measuring the intensity of the solar UVR in the two spectral ranges UV-A and UV-B: 1<380 nm and 1<320 nm are constructed.

- A computer program is worked out for:

- introduction and storage of data from the measurements in the two spectral ranges;

- finding the exact height of the Sun in degrees depending on the hour of measurement as well as the sun’s culmination for each day of the year;

- presenting the data in a table and graphic form.

4. One-year daily data about the intensity of the total solar UVR for the region of Sofia are gathered. They can be taken into account considering the diurnal and yearly course of UVR for the past period.

Conclusion

Regular, regime, instrumental investigations of the solar UVR in Bulgaria are initiated as well as the study of the UV climate and how it is influenced by the set in and continuing unfavorable changes in the ozone layer of the planet. Besides being important on a national scale this start has also its international significance with the view of Bulgaria’s joining the control system of the World Meteorological Organization for investigating UVR and ozone. Together with the fundamental aspect of the investigations, connected with the study of the reasons and factors for the change of UVR and the ozone layer in the region, these investigations are of definite applied and practical character. It is connected with the possibility for suggestions, recommendations and prognosis aiming at the protection from excessive UV radiation in life, economy and tourism.

The station operating since May 1997 is the beginning in a scientific branch whose actuality during the next years will grow. It is considered that in future this station with the accumulated and processed information, methods, etc. will serve not only as a basis for further investigations, but also as a valuable basis necessary for a number of activities connected with the forecast and protection from the harmful effect of the changes in the ozone layer.

## References

Bojkov, R., 1992. Variation of ozone content in the polar regions, WMO. Vol. 41, No 2 (in Russian).

Bojkov, R., 1995. The changing Ozone Layer, Joint WMO and UNEP publication.

Caldwell, L.B., C.W.Camp, W.Warner and S.D.Flint, 1986. Action spectra and their role in assessing 	biological consequences of solar UV radiation in stratospheric ozone reduction, in Solar 	and ultraviolet radiation and plant life, Springer, Heidelberg, pp.87-111.

Lesgegynski K., K.Jokeba, R.Visuri, L.Huurto, J.Simola, T.Koskela, P.Taalas and A.Aarva, 1993. 	Performance test of two Robertson-Berger type UV meters Solar Light model 500 and 501, 	Proc. Int.Symp. on High Latitude Optics, Troms, Norway, 28 June 1993.

Mitchell B.G., 1990. action spectra for ultraviolet photoinhibition of Antarctic phytoplancton and a 	model of spectral diffuse attenuation coefficients in response of marine phytoplancton to 	natural variations in UV-B flux, Proc. of the Scripps Institute of Oceanography, LaJolla, 	Ca, USA, April 1990.

Peak M.J., J.G.Peak and M.Garnes, 1987. Introduction of direct single-strand beaks in human cell 	DNA by far-end near ultraviolet radiations: Action spectrum and Mechanisms. Photocem. 	Photobiol. v.45, pp.381-387.

Petrunov, G., 1976. Regime of the natural UVR in People’s Republic of Bulgaria with the view to its 	use in medical climatology. Extended summary of thesis – Geophysical Institute, BAS (in 	Bulgarian).

Report of WMO Meeting of Experts on UV-B Measurements, Data Quality and Standartization of 	UV Indices. Les Diablerets, Switzerland, 25-28 July 1994.

Robertson D.F., 1982. Solar Ultraviolet Radiation in relation to human sunburn and skin cancer. 	Ph.D Thesis, Univ. of Queensland.

Urbach f. and R.W.Gange, 1986. The biological effect of UVA radiation. Praeger Publishers, New 	York. ch.2.

WMO-WHO Meeting of Experts on Standartization of UV Indices and their Dissemination to the 	Public. Les Diablerets, Switzerland 21-24 July 1997.

Въвеждане на регулярни климатични изследвания на слънчевата ултравиолетова радиация в София

Г.Петрунов, Н.Милошев, Л.Драганов, Г.Милошев

Резюме.  В статията е подробно разгледано поставянето на начало на регулярни инструментални изследвания и измервания на слънчевата ултравиолетова радиация в България. Подробно описани са създадената апаратура и използвани методики. Графично обобщено са представени първите получени резултати за 1997г. В заключение са посочени сферите на бъдещо научно и практическо приложение на получените резултати.