The use of the regulatory PLUME model for the industrial source complex of “Maritza-Iztok” coalmine

N.Gromkova

Geophysical Institute, Bulgarian Academy of Sciences, Atmospheric Physics, Acad.G. Bonchev St., bl. 3, Sofia 1113, Bulgaria, e-mail: gromkova@geophys.bas.bg

Abstract. The dispersion model PLUME has been used to calculate the ground-level concentration for sulphur dioxide (SO2), nitrogen dioxide (NO2), carbon oxide (CO) and dust (PM) from the Maritza-Iztok Ltd. coalmine stacks’ configuration. The meteorological data file, representative for this region, contains sequential hourly records of meteorological parameters, from the month of October 1995 to the month of September 1996. It is shown that the averaged for the all-period pollution fields do not exceed the annual Limit Admissible Concentrations (LAM) for the respective admixtures. The maximum concentration value for all pollutants is evaluated too. The comparison with single LAM shows that only the transient concentration value for sulphur dioxide is higher for the stability class A and wind velocity of 1 m/s.

Key words.  Regulatory dispersion model PLUME, industrial stack, ground-level concentration, sequential hourly meteorological data.

Introduction

The dispersion model PLUME has used a steady-state Gaussian plume equation to model the emission from continuous point sources, such as industrial stacks, and has been created in the Geophysical Institute for a regulatory application (Bulletin of Construction and Architecture, 1998). The model accepts sequential hourly meteorological data records to define the conditions for plume rise, transport and diffusion of the pollutant.

In the present study, the calculation of the ground-level concentration for sulphur dioxide (SO2), nitrogen dioxide (NO2) carbon oxide (CO) and dust (PM) from the Maritza-Iztok Ltd. coalmine stacks has been performed. The source and meteorological input parameters have been provided by the National Center for Environment and Sustainable Development.

- Analysis of the Input Information

- Meteorological input parameters

The meteorological data file, representative for this region, contains sequential hourly records of meteorological variables, from the month of October 1995 to the month of September 1996. Each row consists of :

|  | Column | Parameter sequence | Example |
| --- | --- | --- | --- |
| date | 1 | (YY) year (last 2 digits) | 96 |
| and time | 2 | (MM) month (2 digits from 01 to 12) | 01 |
| of the | 3 | (DD) day (2 digits from 01 to 31) | 03 |
| observation | 4 | (HH) hour (2 digits from 01 to 24) | 01 |
|  | 5 | (WD) wind direction (deg from North) | 110 |
| observed | 6 | (WS) wind speed (m/s) | 5 |
| parameter | 7 | (Tmp) temperature (º K) | 288 |
|  | 8 | (Stb) stability (by Pasquill) | 5 |

Table 1.  Parameter’s sequence in the input meteorological file

In Fig. 1 the wind rose for this period is shown. The rose is drawn up with respect to blow-to wind direction. Each circle represents 5% of the cases in each rose’s ray with such a wind direction:

Fig.  1.  1995/96 wind rose

It is found that the winds are from the west (W) - 22.24%, southwest (SW) - 17.22%, west-southwest (WSW) - 11.07% and south-southwest (SSW) - 7.95%. This means, that the areas of higher concentration of the pollutant should be expected in the northeastern quadrant with respect to the sources’ configuration, depending on their emissions.

Table 2 shows the results of the statistical evaluation of 11 wind categories in 16 wind directions. The wind speeds are between 0 and 1 m/s in 64.07% of the cases. The highest observed speed is 5 m/s. The average speed values of the wind do not exceed 2 m/s. The column and row “total” consisted of 8626 cases, instead of the full number of cases 8784 = 24 hours x 366 day (1996 was a leap year), because in 158 cases the speed values were missing.

The stratification rose is shown in Fig.2. - stability classes in 16 wind directions for this period (drawn with respect to blow-to wind direction). Each circle represents 5% of the stability classes in a given direction.

Fig.  2.  1995/96 rose of the stratification

As is seen, in the predominant wind’s directions - first quadrant of the rose, the stratification is mostly stable - D, E and F classes, i.e. in these directions the concentration will be spread over a longer distance. The situation in the second and in the beginning of the third quadrant is similar - in most cases the stratification is stable. In the rest third and fourth quadrant the stratification is unstable - А, B and C classes, i.e. the diffusion of the pollutants will be achieved very fast and the higher concentration can be found close to the sources. Therefore, the southeastern area of coalmine Maritza-Iztok I, II, и III would be expected to be cleaner than the northwestern one.

In Table 3 the detailed results of statistical processing of the stratification rose are shown. From the last row of the table it is seen that for this period the predominant stratification is E, i.e. the average stratification rose is stable.

Fig. 3 shows the frequency of the wind velocities distribution for a twenty-four-hour period. As is seen in the afternoons - from 1 p.m. until 7 p.m. , the wind speeds are calm (1 m/s) in more than 243 cases.

Fig. 4 shows that in the winter’s months – November, December and January, the wind velocities were 1 m/s - in 74% of the cases. Taken as a whole, the wind speeds do not exceed 5 m/s. This means that during the period October 1995 - September 1996, the meteorological conditions permitted the observation of higher pollutant concentrations.

Fig. 5 shows that the highest temperature of 39 °C is observed during July 1995 and the lowest temperature of minus 6.9 °C - during February 1996. The latter is a very high absolute winter value for this region. The permanent curve shows the average temperature.

Fig. 6 shows the 24-hour stability class frequency. It is known, from the atmospheric dynamic theory, that during the day-time hours (one hour after sunrise until one hour after sunset) the stability classes might be A, B, C or D (neutral state), while during the night-time hours the classes are stable - F, E or D. It can be seen from Fig. 6, 9 a.m. is the first diurnal hour, when the unstable atmospheric conditions begin. For afternoon hours, however, the separation is not so clear - the last daytime hour is 4 p.m. and the first nighttime hour is 9 p.m. The outlined zones 1 and 2 on the figure show, that the observed or calculated solar radiation values are incorrect for the hours between 5–6 p.m. and 7–8 p.m., which leads to the improper stability classes determination. The atypical classes would affect the calculation of the plume rise heights [1]. For the hours 5–6 p.m. these values would be lower and for hours 7–8 p.m. higher than the real ones. All this affected the estimated by Gaussian plume equation hourly concentrations.

| 95/96 | Wind Directions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Speed [m/s] | S | SSW | SW | WSW | W | WNW | NW | NNW | N | NNE | NE | ENE | E | ESE | SE | SSE |  | cases | % |
| 0 - 1 | 173 | 566 | 809 | 565 | 1228 | 358 | 293 | 223 | 394 | 411 | 114 | 69 | 77 | 64 | 88 | 95 |  | 5527 | 64.07 |
| 1 - 1.5 | 36 | 53 | 303 | 207 | 348 | 117 | 83 | 77 | 147 | 147 | 57 | 11 | 15 | 10 | 45 | 47 |  | 1703 | 19.74 |
| 1.5 - 2 | 28 | 37 | 175 | 120 | 152 | 36 | 21 | 1 | 25 | 25 | 14 | 2 | 8 | 7 | 18 | 31 |  | 700 | 8.12 |
| 2 - 2.5 | 20 | 13 | 121 | 42 | 43 | 6 | 0 | 0 | 1 | 5 | 2 | 3 | 1 | 3 | 18 | 35 |  | 313 | 3.63 |
| 2.5 - 3 | 10 | 10 | 42 | 15 | 25 | 1 | 2 | 0 | 4 | 2 | 2 | 0 | 1 | 0 | 9 | 26 |  | 149 | 1.73 |
| 3 - 3.5 | 6 | 3 | 27 | 4 | 4 | 0 | 0 | 0 | 0 | 4 | 1 | 1 | 1 | 0 | 2 | 18 |  | 71 | 0.82 |
| 3.5 - 4 | 7 | 1 | 5 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 9 |  | 28 | 0.32 |
| 4 - 4.5 | 1 | 3 | 3 | 0 | 1 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 2 | 2 |  | 14 | 0.16 |
| 4.5 - 5 | 3 | 0 | 0 | 0 | 116 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | 121 | 1.40 |
| > 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | - |
| u | 1.45 | 1.14 | 1.38 | 1.26 | 1.42 | 1.14 | 1.1 | 1.07 | 1.1 | 1.15 | 1.2 | 1.13 | 1.15 | 1.15 | 1.5 | 1.78 |  |  |  |
| cases | 284 | 686 | 1485 | 955 | 1918 | 518 | 399 | 301 | 571 | 597 | 190 | 86 | 103 | 84 | 186 | 263 |  | 8626 |  |
| % | 3.29 | 7.95 | 17.22 | 11.07 | 22.24 | 6.01 | 4.63 | 3.49 | 6.62 | 6.92 | 2.2 | 1 | 1.19 | 0.97 | 2.16 | 3.05 |  |  | 100 |

Table 2. Wind rose statistical evaluation

| 95/96 | Wind Directions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stb. class | S | SSW | SW | WSW | W | WNW | NW | NNW | N | NNE | NE | ENE | E | ESE | SE | SSE |  | cases | % |
| A | 5 | 4 | 6 | 10 | 55 | 53 | 58 | 49 | 113 | 104 | 18 | 6 | 1 | 0 | 8 | 3 |  | 493 | 5.82 |
| B | 12 | 15 | 67 | 69 | 174 | 115 | 99 | 98 | 151 | 158 | 47 | 10 | 7 | 7 | 13 | 18 |  | 1060 | 12.51 |
| C | 15 | 21 | 106 | 72 | 113 | 70 | 57 | 32 | 81 | 67 | 17 | 12 | 10 | 5 | 14 | 21 |  | 713 | 8.41 |
| D | 71 | 115 | 399 | 249 | 592 | 137 | 91 | 55 | 129 | 127 | 36 | 13 | 30 | 19 | 40 | 83 |  | 2186 | 25.8 |
| E | 27 | 80 | 213 | 100 | 175 | 26 | 27 | 10 | 22 | 35 | 20 | 7 | 13 | 13 | 17 | 33 |  | 818 | 9.65 |
| F | 152 | 450 | 683 | 449 | 732 | 110 | 63 | 52 | 68 | 99 | 47 | 37 | 40 | 39 | 82 | 100 |  | 3203 | 37.8 |
| class | E | E | E | E | E | D | C | C | C | C | D | D | E | E | E | E |  |  |  |

Table 3.  Stratification rose statistical evaluation

Fig. 3.  Scatterplots of 24 hour wind velocity frequency for 1995/96.

Fig. 4.  Scatterplots of monthly wind velocity frequency for 1995/96.

Fig.  5.  Scatterplots of monthly temperature frequency for 1995/96.

Fig.  6.  Scatterplots of 24-hour stratification frequency for 1995/96

- Source input parameters

The input physical and technological stack characteristics are as follows:

| Stack ID | Stack Height [m] | Stack Diameter [m] | Exhaust Gas Temperature [°C] | Exhaust Gas Flow Rate [m3/s] | Pollutant | Actual Emission [t/yr] |
| --- | --- | --- | --- | --- | --- | --- |
| MIz-I-1 | 150 | 6 | 190 | 575.6 | SO2 | 98639.7832 |
|  | 150 | 6 | 190 | 575.6 | NO2 | 3567.8486 |
|  | 150 | 6 | 190 | 575.6 | CO | 165.3349 |
|  | 150 | 6 | 190 | 575.6 | PM | 5801.3645 |
| MIz-I-2 | 120 | 5 | 60 | 433.3 | SO2 | 13296.3488 |
|  | 120 | 5 | 60 | 433.3 | NO2 | 276.3144 |
|  | 120 | 5 | 60 | 433.3 | CO | 22.1048 |
|  | 120 | 5 | 60 | 433.3 | PM | 14688.8269 |
| MIz-I-3 | 120 | 5 | 60 | 288.9 | SO2 | 7998.9952 |
|  | 120 | 5 | 60 | 288.9 | NO2 | 166.2482 |
|  | 120 | 5 | 60 | 288.9 | CO | 13.2996 |
|  | 120 | 5 | 60 | 288.9 | PM | 8836.4640 |
| MIz-II-1 | 325 | 12 | 120 | 2325.6 | SO2 | 386322.5152 |
|  | 325 | 12 | 120 | 2325.5 | NO2 | 15924.1794 |
|  | 325 | 12 | 120 | 2325.6 | CO | 736.7524 |
|  | 325 | 12 | 120 | 2325.6 | PM | 18218.7074 |
| MIz-II-2 | 325 | 10 | 120 | 493.9 | SO2 | 84451.2634 |
|  | 325 | 10 | 120 | 493.9 | NO2 | 3475.8177 |
|  | 325 | 10 | 120 | 493.9 | CO | 160.51408 |
|  | 325 | 10 | 120 | 493.9 | PM | 5860.0502 |
| MIz-III-1 | 325 | 12.4 | 140 | 1960.0 | SO2 | 519344.8087 |
|  | 325 | 12.4 | 140 | 1960.0 | NO2 | 13580.3780 |
|  | 325 | 12.4 | 140 | 1960.0 | CO | 627.7799 |
|  | 325 | 12.4 | 140 | 1960.0 | PM | 9016.2032 |

Table 4.  The input stack parameters for Maritza-Iztok coal-mine

In the first column, the symbols MIz-I-1, 2, 3, MIz-II-1, 2, MIz-III-1 denote the six stacks of Maritza-Iztok coal mine regions: I, II and III respectively.

- Analysis of the Ground-Level Concentration

In Fig. 7 the field (40 x 40 km) of ground-level concentration for sulphur dioxide (SO2) is shown, averaged for the entire period, i.e. yearly.

The black points mark the position of the six Maritza-Iztok coal-mine stacks. The isolines show that the SO2 concentrations do not exceed the annual Limit Admissible Concentration (LAC) of 0.05 mg/m3, which is stated by Bulgarian standards. The maximum concentration of 0.037 mg/m3 is obtained north-northeast of Maritza-Iztok II.

Fig. 8 shows the ground-level concentration for nitrogen dioxide (NO2). The maximum concentration of 0.00083 mg/m3 (with annual LAC of 0.05 mg/m3) is obtained west of the Maritza-Iztok II.

Fig.  7.  SO2 concentration isolines

Fig.  8.  NO2 concentration isolines

Fig.  9.  CO concentration isolines

Fig.  10.  PM concentration isolines

In Fig. 9 the yearly averaged field for carbon oxide (СО) is shown. The value of maximum concentration - 6.8 x 10-5 mg/m3 is obtained north-northeast of Maritza-Iztok.

In Fig. 10 the yearly averaged field for dust (PM) is shown. The value of maximum concentration  (0.0065 mg/m3) is  obtained to the east Maritza-Iztok I. The annual Limit Admissible Concentration for dust is 0.15 mg/m3.

Conclusions

The obtained ground-level concentration (Figs. 7 - 10) for the configuration of Maritza-Iztok coal-mine industrial stacks shows that the averaged for the selected period (October 1995 - September 1996) pollution fields do not exceed the annual Limit Admissible Concentrations for the respective admixtures.

However, the transient concentration values could have been exceeding  the single Limit Admissible Concentrations. In that case, by the model PLUME , the maximal concentration value for all stacks at more disadvantageous meteorological conditions is made - Table 5.

| Admixture | Маx concentration [mg/m3] | Single LAC [mg/m3] | Distance [кm] | Stability class | Wind Speed [m/s] |
| --- | --- | --- | --- | --- | --- |
| SO2 | 1.8381 | 0.5 | 2.4 | А | 1.0 |
| NO2 | 0.0620 | 0.2 | 18.1 | А | 1.0 |
| CO | 0.0029 | 60.0 | 18.1 | А | 1.0 |
| PM | 0.3099 | 0.5 | 2.0 | B | 2.5 |

Table 5.  Maximal Concentrations for Maritza-Iztok coal-mine stacks.

As is seen, only sulphur dioxide (SO2) maximal concentration is 3.68 times higher than the single LAC’s one at stability class А and wind velocity 1 m/s. That’s easy to explain, because the emission of sulphur dioxide is the highest - see Table 4.

## Topical Editor thanks K.Ganev and D.Yordanov for their help in evaluating this paper.

References

Bulletin of Construction and Architecture, 1998 Method of Calculation of the Emitted Construction’s (Stacks) Height, Propagation and the Expected Polluted Admixture Concentrations in the Planetary Boundary Layer, No 7/8 (in Bulgarian)

Оценка на замърсяването от комините на мини “Марица-Изток” чрез регулаторен дифузионен модел PLUME

Н. Громкова

Резюме.  Регулаторният дифузионен модел PLUME се използва за пресмятане приземните полета на концентрация за серен диоксид (SO2), азотен диоксид (NO2), въглероден оксид (CO) и прах (PM) от комините на мини “Марица-Изток” ЕООД. Файлът с метеорологичните данни за този район съдържа последователни почасови записи за тези параметри от месец октомври 1995г. до месец септември 1996г. Показано е, че средно-годишните полета на замърсяване не надминават годишните Пределно Допустими Концентрации (ПДК) за разглежданите замърсители. Максималните концентрации са пресметнати също. Сравнението им с моментните ПДК показа, че само тези за серен диоксид са по-високи при клас на устойчивост A и скорост на вятъра 1 m/s.