REGULATORY MODEL FOR CALCULATING OF VEHICLES’ EMISSIONS, MODULE EMISSIONS OF SOFTWARE TRAFFIC ORACLE - PART I

N. Gromkova

Geophysical Institute, Bulgarian Academy of Sciences, Atmospheric Physics, Acad. G. Bonchev St., bl. 3, Sofia 1113, Bulgaria, e-mail: gromkova@geophys.bas.bg

Abstract.  The software Traffic Oracle is a PC utility tool used for the purposes of the environmental policy in the development of road communications as well as for the impact assessments of vehicle emissions in the surface layer. The method developed, has been funded by the Bulgarian Ministry of Environment and Waters and the project outputs have been approved as a national regulatory standard. The program consists of two main modules – EMISSIONS and DIFFUSION. The first module calculates the emission estimates from the road transport based on the Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook, Third Edition, Road Transport, Copenhagen, 2001. It has been assumed that emissions of burnt-out gases of motor vehicles in their motion along streets and highways can be approximated as continuous linear or area sources with finite length. The module package has been coded in Visual Basic and works under WINDOWS’ 9X, NT or XP. Its structure provides an easy way across the tasks, that the users have to carry out. The input information screen is user-friendly, and has well-provided-to warning messages of missing or restrictions to the supplied data.

Keywords:  regulatory model, air pollution, vehicle emissions, road transport.

Introduction

The product Traffic Oracle has been developed for the purposes of environmental policy in the development of the road communications as well as for the impact assessments of vehicle emissions in the surface layer. The program consists of two main modules – EMISSIONS and DIFFUSION.

The module EMISSIONS calculates the emission estimates from the road transport. It has been assumed that emissions of toxic substances, contained in the burnt-out gasses of motor vehicles in their motion along streets, highways or determinate region (towns or villages), can be approximated respectively as continuous linear sources with finite length or area ones.

The following vehicle category division adopted for description of road transportation is included in the software:

Table 1. Vehicle category split

|  | Categories |  | Categories |
| --- | --- | --- | --- |
| PASSENGER CARS | 1. Gasoline <1.4 l 2. Gasoline 1.4 – 2.0l 3. Gasoline >1.4 l 4. Diesel >2.0 l 5. LPG 6. Two Stroke Gasoline | HEAVY DUTY VEHICLES | 9. Gasoline 10. Diesel 7.5 – 16 t 11. Diesel 16 – 32 t 12. Urban Buses 13. Coaches |
| LIGHT DUTY VEHICLES <3.5 | 7. Gasoline 8. Diesel | TWO WHEELERS | 14. Mopeds & Motorcycles< 50cm³ 15. Motorcycles 50-750 cm³ |

And it covers exhaust emissions of:

Table 2. Pollutants

| NОx | Nitrogen Oxides (NOx: NO and NO2) | Given as NO2 equivalent |
| --- | --- | --- |
| VOC | Volatile Organic Compounds | Given as CH1,85 equivalent (Also -as HC in emission sds.) |
| CH4 | Methane | Given as CH4 |
| CO | Carbon Monoxide | Given as CO |
| CO2 | Carbon Dioxide | Given as CO2 |
| N2O | Nitrous Oxide | Given as N2O |
| SO2 | Sulphur Dioxide | Given as SO2 |
| NH3 | Ammonia | Given as NH3 |
| Cd | Cadmium | Given as Cd |
| Pb | Lead | Given as Pb |
| PAH | PolyAromatic Hydrocarbons (PAHs) including: indeno(1,2,3-cd)pyrene, benzo(a)pyrene, benzo(b)fluoranthene, benzo(k)fluoranthene |  |
| DIOX | Polychlorinated Dibenzo Dioxins (PCDDs) and Polychlorinated Dibenzo Furans (PCDFs) | Given as Dioxins and Furan respectively |
| PCBs | polychlorinated biphenyls |  |
| PM | Particulate Matter | Given as the mass collected on a filter below 52°C in dilution tunnel measurements |

Program Architecture

In the main menu of the EMISSIONS module - Fig.1, there are two options to select, depending on the source type:

- Line Source (roads and highways);

- Area Source (towns and villages).

Both vehicle emission data from line or area sources are provided by two EMEP/CORINAIR calculation schemes - simpler and detailed.

Fig. 1	Start Panel of the Module EMISSIONS

SIMPLER METHODOLOGY

The methodology is based on total fuel consumption data which then have to be multiplied by bulk emission factors, produced by averaging the data submitted by different European countries to CORINAIR - Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook (1996). The simpler methodology covers all vehicle categories (table 1) and pollutants from table 2 except emissions of SO2 и NH3.

DETAILED METHODOLOGY

The methodology is based on Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook (2001). The detailed methodology covers all vehicle categories (table 1) and pollutants from table 2 except emissions of PCBs. The total emissions are calculated by summing emissions from the thermal stabilized engine operation (hot) rate and transient warming-up phase (cold start) for the respective driving situation: urban, rural or highway.

The new emission standards and trends in the evolution of emissions from vehicles have been adopted. The stages of the emissions’ reduction are:

- Conventional vehicle classes - vehicles complying with directive ECE 15/04 (up 1990)

- EURO I - vehicles complying with directive 91/441/EEC (1991-1995)

- EURO II - vehicles complying with directive 94/12/EC (1996-2000)

- EURO III- vehicles complying with directive 98/69/EC Stage 2000

The emission factors for Conventional and EURO I stage for NОX, CО, VOC, PM, CH4 and Fuel consumption (FC) are approximated with a polynomial expression of the speed-dependency equation. The mean speeds which correspond to the driving patterns of vehicles on road classes ‘rural’, ‘urban’ and ‘highway’ for Bulgaria in the range of the approximation validity are:

Table 3. Mean Vehicle speeds

|  | Vehicle categories | urban [km/h] | rural [km/h] | highway [km/h] |
| --- | --- | --- | --- | --- |
| 1 | PC Gasoline <1.4 l | 50 | 90 | 120 |
| 2 | PC Gasoline 1.4 – 2.0l | 50 | 90 | 120 |
| 3 | PC Gasoline >1.4 l | 50 | 90 | 120 |
| 4 | PC Diesel >2.0 l | 50 | 90 | 120 |
| 5 | PC LPG | 50 | 90 | 120 |
| 6 | PC Two Stroke Gasoline | 50 | 90 | 120 |
| 7 | LDV Gasoline | 50 | 80 | 110 |
| 8 | LDV Diesel | 50 | 80 | 110 |
| 9 | HDV Gasoline | 50 | 80 | 110 |
| 10 | HDV Diesel 7.5 – 16 t | 50 | 80 | 110 |
| 11 | HDV Diesel 16 – 32 t | 50 | 80 | 110 |
| 12 | Urban Buses | 50 | 0 | 0 |
| 13 | Coaches | 50 | 70 | 100 |
| 14 | Motorcycles ( > 50 см3) | 50 | 90 | 120 |
| 15 | Mopeds & Motorcycles ( < 50 см3) | 50 | 50 | 0 |

The emission factors of NH3, N2O, DIOX and PAH are constant. The emissions of CO2, SO2, Cd and Pb are fuel dependent and those of SO2 and Pb are calculated on the basis of the related sulphur and lead content in diesel and gasoline fuel respectively.

The emissions of the vehicles and the fuel consumption are affected by the road gradient and in case of heavy duty vehicles and busses the slope influence is much more significant. Therefore a slope correction was incorporated for the mentioned vehicle classes for NОX, CО, VOC, PM and FC emission factors –fig. 2.

Fig. 2	Line Source Emissions by Detailed Methodology

Acknowledgments.  The author is grateful for the scientific collaboration of Dr. N. Miloshev, with whom the physics part of the method is elaborated as well as for his advice in the final software version of the module EMISSIONS.

References

Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook, 1996.

Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook, Third Edition, B710 (Road Transport), Copenhagen, EEA, 2001.

(Received 22.03.2004; accepted 18 June 2004)

Регулаторен модел за изчисляване на емисиите от моторните превозни средства – модул EMISSIONS на програмния продукт TRAFFIC ORACLE - ЧАСТ I

Н. Громкова

Резюме.  Софтуерната програма Traffic Oracle е създадена за целите на екологичната политика при определяне разсейването на емисиите на вредни вещества от превозни средства и тяхната концентрация в приземния атмосферен слой, както и за оценка на въздействието върху околната среда от транспорта. Разработването на методиката беше финансирано от Министерството на околната среда и водите, а резултатите от нея са одобрени за регулаторен национален стандарт. Програмата се състои от два модула - EMISSIONS и DIFFUSION. Първият модул разработен на базата на Joint EMEP/CORINAIR Atmospheric Emission Inventory Guidebook, Third Edition, Road Transport, Copenhagen, 2001, изчислява емисията на определени вредни вещества в отработилите газове от двигателите с вътрешно горене на моторните превозни средства (МПС). Приема се, че емисиите от МПС при движението им по автомагистрали и сложна улична мрежа, могат да се апроксимират като непрекъснато действащи линейни или площни източници. Програмата е създадена с Visual Basic и работи в среда на WINDOWS’ 9X, NT, XP. Логиката в структурата на програмата не затруднява потребителя да изпълни съответната задача и той е улеснен от предупредителните прозорци и съобщения за липсващи или неподходящо въведени входни данни.